# Tankanzeige – TESTEN

TANKNADEL STEHT AUF VOLL – TANK ABER LEER

Einen auf Maximum hängenden Schwimmer kannst Du so bestimmen: Beide Kabel vom Tankgeber ab und dann mit dem Multimeter (auf Ohm stellen) am Tankgeber messen. Sollte der Schwimmer hängen, müsste der Widerstand im Bereich Null bis wenige Ohm sein - vorausgesetzt natürlich, dass Du weißt, dass der Tank eigentlich leer ist!

Bleibt nach dem Abziehen der beiden Kabel vom Tankgeber (& eingeschalteter Zündung!) die Tanknadel immernoch auf voll, dann hast Du irgendwo zwischen den beiden Kabeln einen Kurzschluss.

WEITERES

Hi, The petrol guage in my 1997 Cooper reads empty even when it's 1/4 full. When I fill the car to the brim, it only reads 3/4 full.
Can anyone help?

Many later Minis have this problem, it's to do with the sender wire inside the fuel tank not bent correctly (I think?).

You can also test if the gauge is working properly by shorting the two tank sender wires together with a paper clip. Then with the ignition on the gauge should read full.

Hi Can you tell me how to take the sender out?

First make sure the tank is almost empty or you'll end up with a boot full of unleaded! Disconnect the wires and there is a metal ring round the outside of the sender - tap it anti-clockwise and this will release the sender unit. Be careful not to bend the rod when you take it out

Sender wires are on the side of the thank, round goldy part.
You also need to check if the float inside the tank is still liquid tight ,as sometimes they can split and hold petrol will enter the float and cause the float to sink and give an incorrect reading on the gauge

how do you check that? how do you get to it? you have to remove the sender unit as described theres a thin rod attatched the the rear and on the other end is the plastic float

Hi, my wife and I had the same problem with her 87 Mayfair. One tip for you all make sure when replacing fuel sender unit the rubber seal goes on first before sender unit otherwise you will have a boot full of fuel like we did.