#### Rear Radius Arms, Cone Suspension ( Dry )

No.; Part No.; Description; Qty; Remarks. 1; NAM7162; Swing arm R/H; 1; was 21A392. 1; 21A392/R; Swing arm, R/H, rebuilt; 1. 1; 21A393/c; Swing arm, R/H, complete with brake assembly; 1. 1; NAM7163; Swing arm, L/H; 1; was 21A393. 1; 21A393/R; Swing arm, L/H, rebuilt; 1. 1; 21A393/C; Swing arm, L/H, complete with brake assembly; 1. 2; 2A7351; Radius arm stub axle R/H; 1. 2; 2A7364; Radius arm stub axle L/H; 1. 3; CUD1173; Seal on stub axle for NAM7162/3 only; 2. 4; 2A7352; Stub axle circlip; 2. 5; 21A397; Bracket on radius arm for hose; 2. 6; ACA9099; Pin; 2. 7; ATJ4098; Roller bearing inner, was 13H442; 2. 8; 2A7325; Bronze bush outer; 2. 9; 21A396; Metal grease sleeve, being replaced by plastic sleeve; 2. 10; 2A7324; Outer thrust washer; 2. 11; ACA9302; Inner thrust washer; 2. 12; 2A7327; Sealing rubber ring; 4. 13; LWZ208; Spring washer; 4. 14; FNZ508; Nut; 4. 15; UHN400; Grease nipple; 2. 16; GSV1125; Radius arm repair kit; 2. 17; 21A2023; Hub with studs; 2. 17; 21A1277; Hub with long studs, 'S' and 1275GT plus all 1984 on; 2. 18; 21A2064; Wheel stud, shaft length 1.375" 35mm; 8; Thread length 0.875". 18; 21A2064L; Wheel stud, shaft length 2.000" 50.80mm; 8; Thread length 1.125". 18; ATA7085; Wheel stud, shaft length 1.6875" 42.65mm; 8; Thread length 1.000". 18; 21A1278; Wheel stud, shaft length 2.375" 60.30mm, Cooper S Length; 8; Thread length .750". 18; ENJO-1039; Wheel stud, shaft length 2.375" 60.30mm; 8; Thread length 1.125". 18; ERA1268; Wheel stud, shaft length 2.437" 62mm; 8; Thread length 1.000". 19; 88G322; Wheel nuts, round radius chamfer angle; 16. 19; 113087; Wheel nuts, 60° chamfer angle; 16; Steel wheel 1985 on. 20; CN1; Chrome wheel nut; 16; Alternative to 88G322. 20; CN4; Chrome wheel nut, wide base; 16; Alternative to 88G322. 20; AHA8785; Chrome wheel nut, 60° chamfer dome - 1275GT Rostyle; 16; Weller steel, use CN2. 20; CN2; Chrome wheel nut, 60° chamfer angle; 16; Weller steel and Rostyle. 20; CN5; Chrome wheel nut, wide base; 16; Alternative to CN2. 21; GHS134; Metal backed 'S' oil seal; 2; Use GHS102. 21; GHS102; Hub oil seal; 2. 22; GHB116; Bearing without built in spacer; 4; NLA, use GHK1088. 22; GHB212; Bearing with built in spacer; 4; Part of GHK1088. 23; 2A7354; Spacer when GHB116 fitted (Not Timken); 2. 24; GHK1352; Ball bearing kit with built in spacer and seals; 2; was GHK1352. 24; GHK1548; Timken tapered roller bearing set with seals; 2. 25; BTA672; Hub washer; 2. 26; NL610041; Hub nut R/H; 1. 26; FNN2410; Hub nut L/H; 1; Left hand thread. 27; ATB4098; Grease retaining cap; 2. 28; 21A2454; Hand brake quadrant; 2. 29; CLZ549; Clevis pin, was BH605281 bolt; 2. 30; AWZ105; Washer; 2.