Falschluft-Airleaks

First things to look for when your SPI engine is not running quite right are air leaks. These leaks are usualy the ends of the little black, red and yellow capilary lines, they tend to split as shown below and play havoc with the vacuum reading of the ECU. They should be a very tight fit and nothing less than excellent condition.

Kurz, weiße Markierung ROVER Part Nbr. MLH10022
The main one that seems to cause a problem is the one on the end of the short capilary identified with white, from the back of the inlet manifold to the fuel trap. Located on this port on the back of the inlet manifold, again identified with white.
The fuel trap looks like this, has one short black port and long green port.

Lang, grüne Markierung ROVBEr Part Nbr. MLH10026

While on the subject of capilary tubes and fuel traps, there is a known problem of fuel causing blockage in these tubes, mainly the one identified with green, attached to the long green port of the fuel trap and the ECU at the other end.

The symptoms of this are, hesitation when accelerating usualy acompanied with a backfire in the inlet manifold and the car usualy bucks violently.

Lösung:

The only fix for this is to remove the tubes along with the fuel trap and blow through, remove the ECU and insert some cotton wool into the MAP ( Manifold Absolute Pressure) sensor port and place the whole thing on a warm radiator over night. This is what is suggested in in a Rover service bulletin. Also ensure the capilary ends are in nothing less than excelent condition as above. The MAP sensor port as shown.

Other locations of air leaks are the breather system hoses and the evaperative emissions control system. Check all rubber hoses are in good condition, no cracks or splits. The main breather hoses to check are the ones supplying the purge valve and the charcoal canister. They usualy leak at the purge valve shown here.

| VACUUM PIPE- INLET MANIFOLD TO FUEL TRAP FOR SPI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Price; £2.32 exc vat. £2.73 inc vat. For: Classic Mini Part No. MLH10022. |  |
|  | VACUUM PIPE- FITS TO SPI AIR BOX to THROTTLE BODY(RED) |  |  |  |  |
|  |  |  |  | Price; £2.09 exc vat. £2.46 inc vat. For: Classic Mini Part No. PHP10043. |
|  | Description there are 2 vacuum pipes on the single point injection car.This one fits to the underside of the air box. The other is MLH10022 which has angled ends to fit from fuel trap to inlet manifold |  |  |  |  |

| This item is presently out of stock. See below for any alternative items. |  |  |
| --- | --- | --- |
| FUEL TRAP INTAKE FOR SINGLE POINT INJECTION |  |  |
|  |  | Price; £4.49 exc vat. £5.28 inc vat. For: Classic Mini Part No. NPC10001. |
| Description Fits on the back of the engine as a fuel trap intake and the 2 hoses that attach are MLH10026/7---fits single point injection cars only |  |  |

| VACUUM PIPE WITH CONNECTORS |  |  |
| --- | --- | --- |
|  |  | Price; £2.20 exc vat. £2.59 inc vat. For: Classic Mini Part No. 37H4229M. |
| Description Thin plastic pipe with rubber connectors either end-This replaces the old metal type which also had a vacuum chamber built into the line, that was made obsolete. This hose is 26"(660mm)long and has 2 rubber ends,one staright and one at a right angle. |  |  |