Electrical - Battery Specifications

A standard Mini battery is a Type 038.

Type 038
Capacity: 38Ah
CCA(SAE): 360A
RC: 50 Minutes

Dimensions(mm)
245 x 135 x 200
210 x 175 x 175

Other larger batteries will also fit in the standard Mini battery box and will help starting on cold mornings and wont go flat so quickly if you leave your stereo / lights on. Type 063 and Type 065 are good candidates.

Type 063 
Capacity: 44Ah
CCA(EN): 330-440A
RC: 70 Minutes

Dimensions(mm)
212 x 175 x 175

Type 065
Capacity: 53Ah
CCA(EN): 470A
RC: 90 Minutes

Dimensions(mm)
242 x 175 x 175

Cold Cranking Amps (CCA)
This is the number of amps a new, fully charged car battery can supply for 30 seconds, while maintaining a voltage of at least 7.2 volts.

The higher the CCA rating, the greater the starting power of the battery.

Reserve Capacity (RC)
Represents the number of minutes a 12 volt battery can be discharged at 25 amps and maintain a voltage of 10.5 volts. The higher the RC, the longer the car battery can operate should its alternator or fan belt fail.

Ratings and Sizes are approximate, based on leading brands and may vary.